AI-DOE Parameter Space Definition Script

Here is a Python script that defines the parameter space for the AI-Design of Experiments (AI-DOE) simulation of the PBA alternative. This script is based on the technical specifications found in the Q-SMEC Deep Scientific Analysis_text.txt and MASTER_VALIDATION_DATABASE.txt files. It uses standard libraries to generate the full factorial design space, which can then be fed into a simulation workflow.

python

import itertools

import pandas as pd

from typing import List, Dict, Any

def define_pba_parameter_space() -> Dict[str, List[Any]]:

"""

Defines the parameter space for the Q-SMEC PBA alternative AI-DOE simulation.

Based on the 'Q-SMEC Deep Scientific Analysis_text.txt' document, which specifies

approximately 8 controllable factors, each with 3 levels. This function

establishes a representative parameter set for optimizing a PBA material.

Returns:

A dictionary where keys are parameter names and values are lists of

the three levels for that parameter.

"""

# The documentation mentions 10 disclosed elements. For a PBA (Iron Hexacyanoferrate),

# the core is Fe-C-N. We select other documented elements as potential dopants or

# structural modifiers relevant to battery chemistry.

# Elements chosen from the disclosed list: Ge, Ga, Sb, Te, Se, Al, Nb, N, Sn, Ti

parameter_space = {

# Factor 1: Primary Cation Dopant (replaces some Fe)

"primary_dopant_element": ["Al", "Ga", "Ti"],

# Factor 2: Dopant Concentration (log scale is common for wide ranges)

"dopant_concentration_cm3": [1e18, 1e19, 1e20],

# Factor 3: Cyanide Vacancy Control (affects ion channels)

"cn_vacancy_percentage": [0.5, 1.5, 3.0],

# Factor 4: Interstitial Water Content (x in Fe4[Fe(CN)6]3·xH2O)

"water_content_x": [4, 10, 16],

# Factor 5: Deposition Temperature for CVD (affects crystallinity)

"cvd_temperature_c": [350, 475, 600],

# Factor 6: Annealing Duration (affects grain size and strain)

"annealing_duration_min": [30, 60, 120],

# Factor 7: Annealing Temperature (finalizes crystal structure)

"annealing_temperature_c": [250, 400, 550],

# Factor 8: Layer Thickness (affects total capacity and resistance)

"smec_layer_thickness_um": [5, 12, 20]

}

return parameter_space

def generate_doe_runs(param_space: Dict[str, List[Any]]) -> pd.DataFrame:

"""

Generates a DataFrame containing all factorial combinations for the DOE.

Args:

param_space: A dictionary defining the parameter space.

Returns:

A pandas DataFrame where each row is a unique simulation run and

each column is a parameter.

"""

keys = param_space.keys()

values = param_space.values()

# Create the Cartesian product of all parameter levels

doe_combinations = list(itertools.product(*values))

# Create a DataFrame for clarity and easy export

df_doe = pd.DataFrame(doe_combinations, columns=list(keys))

df_doe.index.name = "RunID"

return df_doe

if __name__ == '__main__':

# 1. Define the parameter space for the PBA alternative

pba_params = define_pba_parameter_space()

# 2. Generate the full factorial (3^8 = 6,561) runs

doe_runs_df = generate_doe_runs(pba_params)

# 3. Display summary and a sample of the runs

print("="*50)

print("Q-SMEC AI-DOE Parameter Space for PBA Alternative")

print("="*50)

print(f"Total Parameters (Factors): {len(pba_params)}")

print(f"Levels per Parameter: {len(next(iter(pba_params.values())))}")

print(f"Total Simulation Runs: {len(doe_runs_df)}")

print("\n--- Parameter Space Definition ---")

for param, levels in pba_params.items():

print(f"- {param}: {levels}")

print("\n--- Sample of Generated DOE Runs (First 5) ---")

print(doe_runs_df.head())

print("\n--- Sample of Generated DOE Runs (Last 5) ---")

print(doe_runs_df.tail())

# 4. (Optional) Save the full DOE run list to a CSV file

# output_filename = "pba_doe_simulation_plan.csv"

# doe_runs_df.to_csv(output_filename)

# print(f"\nFull DOE plan saved to '{output_filename}'")

Explanation of the Code

- define_pba_parameter_space(): This function creates a dictionary representing the 8 factors and 3 levels for the AI-DOE, as specified in the documents. The factors chosen are directly relevant to optimizing a PBA material for battery applications:

  - Doping Element & Concentration: To tune electronic conductivity and redox potential.

  - CN Vacancy & Water Content: To engineer the "Matrix Diffusion Channel Size" for better ion transport.

  - CVD/Annealing Parameters: To control crystallinity, grain size, and "Strain Elimination".

  - Layer Thickness: To balance capacity against internal resistance.

- generate_doe_runs(): This function takes the parameter space and uses Python's itertools.product to generate the Cartesian product of all parameter levels. This efficiently creates all 6,561 unique combinations for the full factorial experiment. The result is loaded into a pandas.DataFrame for easy viewing and manipulation.

- if __name__ == '__main__':: The main block executes the functions, prints a summary of the generated experimental plan, and shows the first and last five runs as a sample. It also includes a commented-out line to save the entire plan to a CSV file, which would be the input for the subsequent simulation stage (e.g., jobs submitted to a high-performance computing cluster running Psi4, CP2K, etc.).

This script provides a clear, well-documented, and executable starting point for the AI-DOE (Task 2) of the PBA alternative development plan.