# VALIDATION REPORT: PERSONAL SYSTEM (CAPE TEST)

System: Personal foam fractionation purifier (1–10 GPD) with QSMEC telemetry
Test Campaign: Cape facility, 30-day continuous (24/7) run
Date: Completed December 2025
Authors: Field Test Team + QA

## Objectives

- Verify bio/chem removal efficacy in fresh and saltwater.

- Measure uptime, energy use, and maintenance load.

- Validate QSMEC 8-sensor predictive maintenance model.

- Confirm durability under mud/silt and intermittent power.

## Configuration

- Flow: 1–10 gallons/day variable.

- Power: 50–150W, mix of wall, vehicle, and 200W solar + 400Wh battery.

- Sensors (QSMEC): turbidity, TDS, pH, temp, pressure, flow, VOC proxy, UV intensity.

- Consumables: Foam media cartridges; UV lamp (rated 10,000 hrs).

- Deployment: Backpack/MOLLE; 2-minute setup.

## Results

- Uptime: 99.5% over 30 days (36 minutes downtime total).

- Energy: 0.05–0.15 kWh/gal (3–5× better than RO).

- Efficacy: >99.9% biological removal; >95% chemical removal; <0.5 NTU.

- Throughput: Met 1–10 GPD envelope across test days.

- Predictive Maintenance: 50% downtime reduction vs. non-predictive baseline; alert precision 82%, recall 76%.

- Filter Life: 500–1,000 gal observed range; no membrane fouling.

- Environmental: Operated in sand/mud; no performance loss after 50 on/off cycles.

## Issues & Resolutions

- Voltage Sag (battery): Single dip to 10.8V; added low-voltage cutoff and alert; no recurrence.

- Salt Spray on Connectors: Minor corrosion spotting; added conformal coat + gaskets.

- Sensor Drift (TDS): 2.5% drift after day 18; calibrated in-field; added self-check cadence every 72 hours.

## Data Package

- 500+ sample analyses logged (CSV + telemetry).

- QSMEC model weights + inference notebook (edge-convertible).

- Test scripts and raw maintenance log (JSON).

- Photographic evidence of setups and mud/silt runs.

## Compliance Path

- Target Standards: NSF P248, MIL-STD-810H profiles (temperature, humidity, sand/dust, shock).

- Medical: Align with TB MED 577 potable water guidelines.

- Cyber/Telemetry: Edge-first; optional TLS uplink; no PII; local-only default.

## Conclusions

The personal system met or exceeded all success criteria: high efficacy on bio/chem threats, dual fresh/saltwater performance, low energy, and 99.5% uptime. Predictive maintenance is validated with meaningful downtime reduction. Remaining actions: finalize P248 test scheduling, extend sensor drift study to 60 days, and document corrosion mitigation in the production BOM.