# TECHNICAL DOSSIER: PERSONAL WAR-FIGHTER WATER SYSTEM

System: Personal foam fractionation purifier with integrated QSMEC telemetry
Throughput: 1–10 GPD (field-selectable)
Power: 50–150W (battery/solar/vehicle/wall)
Weight/Size: <5 lbs; backpack/MOLLE compatible
TRL: 6–7 (Cape test validated)
Date: January 2026

## Core Architecture

- Intake & Pre-Filtration

  - Coarse mesh + particulate separator.

  - Handles mud/silt; no membrane fouling.

- Foam Fractionation Reactor

  - Removes biologicals and organics; high surface-area foam media.

  - Saltwater-compatible; no salinity penalty.

- UV Polishing

  - 254 nm UV; rated 10,000 hrs; inline.

- QSMEC Sensor Suite (8x)

  - Turbidity, TDS, pH, temperature, pressure, flow, VOC proxy, UV intensity.

  - Edge processing for predictive maintenance.

- Power Subsystem

  - Accepts 12–24V DC; inverter for AC.

  - Works with 200W solar + 400Wh battery kits; vehicle DC; wall AC.

## Performance Metrics

- Removal: >99.9% biological; >95% chemical; <0.5 NTU.

- Energy: 0.05–0.15 kWh/gal (3–5× lower than RO).

- Flow Envelope: 1–10 GPD across fresh and saltwater.

- Uptime: 99.5% (30-day Cape).

- Filter Life: 500–1,000 gal; UV 10,000 hrs.

- Maintenance: 2-minute filter swap; weekly rinse; monthly sensor check.

## Control & Telemetry

- Modes: Normal, Boost (high flow), Low-Power (battery/solar).

- Alerts: Pressure drop, turbidity spike, VOC excursion, UV intensity low, battery low, flow deviation.

- Predictive: Edge model forecasts clogging and UV degradation; targets 50% downtime reduction.

- Interfaces: Local LEDs + haptics; optional mobile app “War-Fighter H2O” (BLE/LoRa where allowed).

## Environmental & Ruggedization

- Operating Range: -20°C to 50°C; IP65 target.

- Sand/Mud: Protected connectors; conformal coat; gasketed enclosure.

- Shock/Vibration: MIL-STD-810H profiles planned; backpack drop tested to 1.5 m.

## Safety & Compliance

- Standards Target: NSF P248; MIL-STD-810H (temp, humidity, sand/dust, shock); TB MED 577 alignment.

- Materials: Potable-safe wetted surfaces; no PFAS additives.

- Cyber: Edge-first, no PII; optional TLS uplink; secure boot planned for production.

## BOM Highlights (COTS Where Possible)

- Foam media cartridges (field swappable).

- UV lamp assembly (10k hr).

- Brushless blower + DC pump (sealed).

- Sensor set (8x) with conformal coating.

- Battery/solar harness + low-voltage cutoff.

- Enclosure with quick-access panel; Berry-compliant fabrics for harness.

## Production Path

- Pilot Build: 25–50 units (Q2–Q3 2026).

- LRIP: 1,000 units (2027) with dual-source consumables.

- FRP: 10,000 units (2028–2029) via prime partner.

- QA: 100% functional test; batch water-quality sampling; telemetry QA.

## Open Items

- Finalize P248 test lab schedule and budget.

- Extend sensor drift study to 60–90 days.

- Lock mobile app UX for offline-first logging.

- Document corrosion mitigations in BOM and maintenance card.