# QSMEC SPEC: PERSONAL SYSTEM TELEMETRY & PREDICTIVE MAINTENANCE

System: Personal foam fractionation purifier (1–10 GPD)
Module: QSMEC (Quantum Sensor Monitoring & Edge Computing)
Date: January 2026

## Objectives

- Provide real-time water quality and health-of-system telemetry.

- Predict maintenance needs to reduce downtime ≥50% vs. non-predictive baseline.

- Operate edge-first with optional uplink; no PII captured.

## Sensor Suite (8x)

- Turbidity (NTU)

- TDS / conductivity (ppm)

- pH

- Temperature (°C)

- Pressure (psi)

- Flow rate (GPH)

- VOC proxy (gas sensor)

- UV intensity (lamp health)

## Edge Processing

- Algorithms: Moving averages, thresholds, anomaly scoring, and a small predictive model for clogging/UV degradation.

- Targets: Alert precision ≥80%, recall ≥75% (Cape achieved 82/76).

- Sampling: 1 Hz default; burst mode for diagnostics.

- Power Budget: <2W average for sensing + compute.

## Alerts & Actions

- Clogging Forecast: Rising pressure drop + flow decay → pre-emptive filter swap suggestion.

- Water Quality: Turbidity/TDS/VOC excursions; pH out-of-range; UV low intensity.

- Power: Battery low-voltage cut; energy mode switch.

- Environment: Over-temp/under-temp protections.

## Interfaces

- Local: LEDs + haptics; button to acknowledge alerts.

- App (Optional): “War-Fighter H2O” for offline-first logging; BLE/LoRa where allowed; exports CSV/JSON.

- Data Model: Timestamps ISO 8601; JSON records with sensor payloads and alert events.

## Security & Data Handling

- Edge default (no uplink required).

- TLS when uplink enabled; no PII; device ID only.

- Secure boot and signed firmware planned for production.

## Testing & Validation

- Cape Results: 50% downtime reduction vs. non-predictive; 99.5% uptime overall.

- Drift Checks: Auto-cal every 72 hours for TDS/pH; UV self-check on startup.

- Environmental: Conformal-coated PCBA; gasketed enclosure; tested under sand/mud exposure.

## Roadmap

- Expand model to incorporate ambient weather forecasts for pre-positioned maintenance.

- Add optional satellite/SIGINT-friendly low-bandwidth mode (store-and-forward).

- Integrate over-the-air signed updates; field-disable uplink for OPSEC missions.

- Publish API spec for primes to ingest telemetry into soldier systems backends.